# CORN PROCESSING SYSTEM

Sequence of Operation for CompactLogix 5380

# 1. SYSTEM OVERVIEW

The system consists of 4 Material Groups separated by 3 Pneumatic Gates. Material flows forward, but control logic operates in reverse (pull system) - triggered by Receiver's START button.

## Material Flow Diagram

[GROUP 1] → GATE 1 → [GROUP 2] → GATE 2 → [GROUP 3] → GATE 3 → [GROUP 4]

Feeder Tray → Sheller/Conveyor/Aspirator → VMEK → Seed Treater R12

(Material_Gate) → (Aspirator_Hooper_Gate) → (R12_Hopper_Gate)

⚠ CRITICAL: Only ONE material per group at any time. Groups must be EMPTY before receiving new material.

# 2. GROUP DEFINITIONS

| Group | Location | Equipment | Exit Gate |
| --- | --- | --- | --- |
| 1 | Feeder Tray | Bandeja + Barcode Scanner | Material_Gate |
| 2 | Shelling + Transfer | AEC Sheller → Air Conveyor → Aspirator → Aspirator Hopper | Aspirator_Hooper_Gate |
| 3 | VMEK Processing | VMEK (Color Sorter + Counter) → R12 Hopper | R12_Hopper_Gate |
| 4 | Seed Treatment | Seed Treater R12 → Envelope | — (Final) |

# 3. CONTROL LOGIC (PULL SYSTEM)

The Receiver operator controls the system with the START button. When pressed, system checks conditions from Group 4 backwards to Group 1.

## START Button - ENABLED when:

- Group 4 (Seed Treater) = EMPTY

- R12_Ready = TRUE

- R12_Envelop_Present_Sensor = TRUE

- Compressed_Air = TRUE

- No active faults

## Cascade Sequence (on START)

- Open R12_Hopper_Gate: Transfer Group 3 → 4

- When Group 3 empty: Open Aspirator_Hooper_Gate, transfer Group 2 → 3

- When Group 2 empty: Open Material_Gate + Air_Conveyor + Aspirator_Gate

- Group 1 empty: Loader can load new material

# 4. GATE CONTROL CONDITIONS

| Gate | Open Conditions | Close Conditions |
| --- | --- | --- |
| Material_Gate (Group 1→2) | • Group 2 = EMPTY • Material in Group 1 • Compressed_Air = TRUE | • AEC_Material_Sensor = FALSE (no material for 3 sec) |
| Aspirator_Hooper _Gate (Group 2→3) | • Group 3 = EMPTY • VMEK = READY (OPC) • Material in Group 2 • Compressed_Air = TRUE | • Aspirator_Hopper_Sensor = FALSE (no material for 3 sec) |
| R12_Hopper_Gate (Group 3→4) | • Group 4 = EMPTY • R12_Ready = TRUE • R12_Envelop_Present = TRUE • START button pressed • Compressed_Air = TRUE | • R12_Hopper_Sensor = FALSE (no material for 3 sec) |

## Air Conveyor & Aspirator Gate (Synchronized)

Air_Conveyor and Aspirator_Gate operate together - both ON when material transfers through Group 2, both OFF when complete.

# 5. PLC I/O MAPPING

Hardware: CompactLogix 5380 (192.168.101.96)

## Slot 1: Digital Inputs (5069-IB16)

| IN | Tag Name | Address |
| --- | --- | --- |
| 0 | AEC_Sheller_ON | Local:1:I.Pt00.Data |
| 1 | Compressed_Air | Local:1:I.Pt01.Data |
| 2 | Dust_Colletor_ON | Local:1:I.Pt02.Data |
| 3 | Seed_Treater_R12_Running | Local:1:I.Pt03.Data |
| 4 | Material_Gate_Open_Pos | Local:1:I.Pt04.Data |
| 5 | Material_Gate_Close_Pos | Local:1:I.Pt05.Data |
| 7 | AEC_Material_Sensor | Local:1:I.Pt07.Data |
| 8 | Aspiration_Gate_Open | Local:1:I.Pt08.Data |
| 9 | Aspiration_Gate_Close | Local:1:I.Pt09.Data |
| 10 | Aspirator_Hopper_Sensor | Local:1:I.Pt10.Data |
| 11 | R12_Hopper_Sensor | Local:1:I.Pt11.Data |
| 12 | R12_Gate_Open_Pos | Local:1:I.Pt12.Data |
| 13 | R12_Gate_Close_Pos | Local:1:I.Pt13.Data |
| 14 | R12_Ready | Local:1:I.Pt14.Data |
| 15 | R12_Envelop_Present_Sensor | Local:1:I.Pt15.Data |

## Slot 2: Digital Inputs (5069-IB16)

| IN | Tag Name | Address |
| --- | --- | --- |
| 0 | Aspirator_Hooper_Gate_Close_Pos | Local:2:I.Pt00.Data |
| 1 | Aspirator_Hooper_Gate_Open_Pos | Local:2:I.Pt01.Data |
| 2-15 | SPARE | Local:2:I.Pt02-15.Data |

## Slot 3: Digital Outputs (5069-OB16)

| OUT | Tag Name | Address |
| --- | --- | --- |
| 0 | AEC_Sheller_Run_Command | Local:3:O.Pt00.Data |
| 1 | AEC_Sheller_Stop_Command | Local:3:O.Pt01.Data |
| 2 | R12_Permit | Local:3:O.Pt02.Data |
| 3 | Material_Gate_Open | Local:3:O.Pt03.Data |
| 4 | Air_Conveyor | Local:3:O.Pt04.Data |
| 5 | Aspirator_Hopper_Gate | Local:3:O.Pt05.Data |
| 6 | Aspirator_Gate | Local:3:O.Pt06.Data |
| 7 | R12_Hopper_Gate | Local:3:O.Pt07.Data |

## Ethernet Devices

| IP Address | Equipment |
| --- | --- |
| 192.168.101.96 | PLC (CompactLogix 5380) |
| 192.168.101.102 | PanelView 1 (Loader HMI) |
| 192.168.101.103 | PanelView 2 (Receiver HMI) |
| 192.168.101.104 | Printer (Zebra Label) |
| 192.168.101.105 | VMEK (OPC UA) |
| 192.168.101.106 | R12 Seed Treater |
| 192.168.101.100 | Edge Gateway Optix |

# 6. EMERGENCY STOP

System stops when:

- Compressed_Air = FALSE

- Dust_Colletor_ON = FALSE

- OPC connection lost to VMEK

- Manual E-STOP pressed

Actions:

- All gates HOLD current position

- Disable START button

- Display fault on HMI

- Require supervisor override to restart